Stand Alone MCU Programming Conventions

Overview: A program for the Stand-Alone MCU is a standard text file (with extension .txt) that contains a series of one-line commands separated into different sections. Program files are edited on a PC and transferred to the MCU via USB stick.  Program files should be stored on the USB in a root folder entitled “FLICPrograms” (case sensitive). All files with the “.txt” extension will be transferred from this folder to the MCU when the Programs->Custom->Get (USB) button is pressed. Programs transferred to the MCU in this way are available for loading in the Programs->Custom screen. The programming interface is actively evolving based on feedback from our users.  Please contact Scott Pletcher (spletch@umich.edu) to obtain the latest information and to submit questions.

General Syntax:

Each line of a program consists of either a single section definition or a single command.  Commands are separated into a command name and set of parameters, separated by a colon.  Command names are not case sensitive but must be spelled correctly. Parameters must be provided as expected, either a single integer (Duration, Frequency, Pulse width, Decay, Delay, and MaxTimeOn), an array of integers separated by commas (Interval or Linkage), a single word (Program Type or Baseline), or a military time as HH:MM:SS (Start Time).

Syntax errors are not explicitly reported. If there are one or more errors in the program (syntax, value errors, etc.) than the program will simply not load and will not be presented in the program screen.

Command Definitions:

StartTime:  This is the defined start time of the experiment, expressed in military time. Default time is set at the time the program is loaded.  This value, together with the ExpDurationMin parameter, determine the end time of the experiment. Format: HH:MM:SS.

ExpDurationMin: This is the duration, in minutes, from the start time of the experiment until recording will be halted. This parameter should always be explicitly defined. Default (min): 1.  Range: 1-232.

OptoFrequency: This is the frequency (in hz) that the LEDs in the Optolid will pulse when activated. Default: 40hz. Range (hz): 1-500.

OptoPW: This is the pulse width (in milliseconds) representing the on-time for each pulse. It must not equal or exceed the duration of a single period. Default: 8ms. Range (ms): 1-(1000/OptoFrequency).

OptoDecay: This is the duration (in milliseconds) in which an LED on the Optolid will remain active after conditions of activation have ended. Default: 0ms. Range (ms): 0-65536.

OptoDelay: This parameter, if non-zero, can be used to define two different types of reinforcement, based on the value of MaxTimeOn.  If MaxTimeOn=0, then OptoDelay is the fixed delay in a standard fixed interval paradigm. If MaxTimeOn>0 then OptoDelay is the duration (in milliseconds) in which an LED on the Optolid will held from being activated following the termination of conditions of activation. In both cases, the duration of activation is determined by the OptoDecay parameter. Default: 0ms. Range (ms): 0-65536.

MaxTimeOn: This is the duration (in milliseconds) in which an LED is allowed to remain continuously active, after which it will be forced off regardless of whether signal currently exceeds activation threshold. Default: 0ms (no limit). Range (ms): 0-65536.

Linkage: This array of integers (one integer per well) defines whether different LEDs on the OptoLid will be linked in their behavior.  LEDs marked with the same number will be combined with an operational “or” to determine activation, such that all will be activated if one or more are activated. Default (no linkage): {1,2,3,4,5,6,7,8,9,10,11,12}

Baseline: This will indicate whether baseline DFM signals are removed at the start of the experiment and prior to determining activation thresholds.  Default: Yes. Range: Yes/No.

ProgramType:  This value determines how program intervals are sequenced and interpreted.  Allowable values are ‘Linear’, ‘Repeating’, ‘Circadian’, and ‘Constant’. Linear programs proceed through the defined interval steps and remain at the last step until the end of the experiment. Repeating programs return to the first interval once the last defined interval has completed.  Circadian programs are a special case of repeating programs that are held to the condition that the cumulative duration of all intervals is equal to 1440min (i.e., one 24-hour cycle). Constant programs execute only the first defined interval.

Interval: When defined for DFMs, an interval command defines the state of the DFM (running dark or not) and the threshold values for each feeding well, which are used to trigger LEDs on the OptoLid. When defined for the Environmental Monitor, an interval command defines the state of the EVM (running dark or not) and the desired states of the two relays and four output pins (on/off).

For DFMs, each Interval command is followed by an integer array comprised of 14 integers, separated by commas. The first integer indicates whether the DFM will “run dark,” which means that the four indicator LEDs on the top of the board will be turned off and will not indicate their respective status (e.g., heartbeat, communication, etc). Any non-zero value will indicate dark running.  The next 12 values represent threshold values for each of the twelve wells on the DFM (listed in row major order, 1A, 1B, 2A, 2B, etc.). When a signal from the feeding well exceeds this value, its LED (and any linked LEDs) will be activated. A zero threshold indicates constant activation (irrespective of the signal value) and a -1 threshold indicates constant deactivation. The final integer is the duration of the interval in minutes. Default: NA. Range: NA.

For the Environmental Monitor, the same format is used, although the threshold values have different meaning. As for DFMs, the first integer defines dark running. The next six integers set the states of the relays and pins, which can be used to control external devices.  A non-zero value indicates the corresponding relay/pin is set to “on”, while a zero value indicates the relay/pin will be turned “off.”  The order is as follows: Relay1, Relay2, Output1, Output2, Output3, Output4.  The following six integers are ignored.  The final integer is the duration of the interval in minutes, as for the DFMs. Default: NA. Range: NA.

#: Indicates a comment.  Lines beginning with this character will be ignored by the MCU when loading a program.

Sections:

Programs are divided into sections, which are delineated by section names in brackets:

[General] – This indicates that commands and parameters that follow will be applied globally and will be used as default values when values for individual DFMs are not specified. Commands StartTime, ExpDurationMin, and Baseline are only allowed in the General section, and they always apply to all DFMs in the experiment.

The following General section will set global values in which the experiment is set to begin recording at 1:13am.  The experiment will record for 120min (and therefore end recording at 3:13am), and Interval commands will be executed linearly, with the last Interval remaining active until program end.  When individual LEDs on the Optolid are activated, they will pulse at 30hz with a pulse width of 8ms.  This corresponds to a duty cycle of roughly 24% (each period is roughly 33ms). There will be no delay, so an LED will activate immediately after the signal exceeds the defined threshold value.  The decay is 500ms so the LED will remain active for ½ second after the signal decreases below the threshold value.  If a signal continuously exceeds threshold for more than 2.5 seconds, it will be forced off by the MaxTimeOn value. LEDs 2 and 3 share the same linkage value, indicating that both LEDs will be activated when either is above threshold.  Finally, DFM values will be “baselined” at the start of the experiment so that no fly interaction is set to a value of 0.

[General]

StartTime: 1:13:00

ExpDurationMin: 120

ProgramType: Linear

OptoFrequency: 30

OptoPW: 8

OptoDelay: 0

OptoDecay: 500

MaxTimeOn: 2500

Linkage: 1,2,2,4,5,6,7,8,9,10,11,12

Baseline: Yes

Not every command is required.  Commands that are not listed have their global values set to their corresponding defaults. For example,

[General]

ExpDurationMin: 120

Baseline: Yes

will define an experiment whose start time is set to the time the program is loaded (or reset by the user when the “Now” button is pressed on the program screen).  Program type will be linear. OptoFrequency is set at 40hz and OptoPW is set at 8ms. OptoDelay, OptoDecay, and MaxTimeOn are all set at zero. And Linkage is defined such that all LEDs act independently.

[DFM] – This indicates that commands and parameters that follow will be applied to a specific DFM. Any specified values will override the global values set in the General section. Commands not listed will be set to the corresponding global value(s).  The DFM section has one command that is unique to it, which is the ID command.  The ID represents the ID of the DFM to which the Section and the commands pertain. The ID is the same number chosen by the selectors on the DFM and used to indicate DFM “number” on the MCU data screen. (Range=1-98, The ID=99 is reserved for the Environmental Monitor).

The following DFM section will apply to DFM #1 and define a linear program with three intervals, each of 120min in duration.  If the total ExpDurationMin defined in the general section is less than 360min, then the program will end during one of the defined intervals (in other words, the defined experiment duration takes precedent and extra intervals are ignored).  If the ExpDurationMin is set longer than 360 minutes, than the final interval will remain in effect until the end of the experiment (if the ProgramType was “Repeating” then the first interval would become active again after the last one expired). The linkage specifies that each LED will act independently, which is common for single well feeding chambers.

[DFM]

ID: 1

ProgramType: Linear

Linkage: 1,2,3,4,5,6,7,8,9,10,11,12

Interval: 1,20,20,20,20,20,20,20,20,20,20,20,20,120

Interval: 0,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,120

Interval: 1,0,0,0,0,0,0,0,0,0,0,0,0,120

For the first interval of 120min, the DFM will run dark and the OptoLid LEDs will be activated for each well independently if that well’s signal exceeded 20. Values defined in the General section and not redefined here will apply.  So, given the first general section above, this DFM will pulse LEDs at 30hz, for 8ms per pulse with no delay and a ½ second decay.

During the second interval (experiment time 120min-240min) the DFM will not be dark running, so the indicator LEDs on the surface of the DFM will be active. All thresholds are set to -1, so LEDs will be forced off throughout this interval regardless of the feeding signals.

During the final interval, the DFM will again run dark, and all LEDs will be set to pulse continuously until the end of the experiment.

The Linkage parameter provides flexibility to the LED behavior, and it is most often used to link LEDs in choice chamber experiments.  In these experiments a single chamber contains two feeding wells (one on the right and one on the left), and often the experimenter would like both LEDs in the chamber to activate following an interaction with one specific food.  For example, the following DFM section would activate both LEDs in a choice arena only when individual flies interact with the food on the right side of the chamber (i.e., the ‘B’ wells). An OptoFrequency of 10hz would apply to this DFM and override the value defined in the General section.  Other opto-parameters would remain unchanged.

[DFM]

ID: 1

ProgramType: Linear

OptoFrequency: 10

Linkage: 1,1,3,3,5,5,7,7,9,9,11,11

Interval: 0,-1,20,-1,20,-1,20,-1,20,-1,20,-1,20,120

Linkage can also be used to provide optostimulation to control flies in separate wells to maintain identical time and duration as treatment flies.

Note: If a DFM is present in the experiment but it is not referenced in a loaded experiment file, a simple default DFM section will be added to the program. It will be of the following format (using DFM #3 and ExpDurationMin=120, as an example).

[DFM]

ID: 3

ProgramType: Linear

Interval: 0,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,120

[ENVMON] – This indicates that commands and parameters that follow will be applied to the Environmental Monitor (Version 3).

The format of this section mirrors that used for the DFM but (as described above) only the first six threshold values are used to define the states of the Relays and Output Pins.  An ID command is not needed because the ID of the Environmental Monitor is fixed in the firmware to 99. Also, opto-parameters and linkage do not apply because output pins are not pulsed.

The following ENVMON section defines two intervals of 120min each.  During the first 120min, the monitor will not be running dark. Relay 1 will be triggered to the “on” position, while Relay 2 will not.  Output Pins 1 and 2 will be set high.  During the second interval (beginning at 120min after the start of the experiment), both relays will be turned “off” and Output Pins 3 and 4 will be set “high.” Output Pins 1 and 2 will be set “low.”  Because the ProgramType is linear, the second interval command will apply after 120min until the end of the experiment.

[ENVMON]

ProgramType: Linear

Interval: 0,1,0,1,1,0,0,0,0,0,0,0,0,120

Interval: 1,0,0,0,0,1,1,0,0,0,0,0,0,120

Sample Experiment:

The program file “OpenClosedLoop.txt” (in the FLICPrograms folder) has DFM 1 following an “open-loop” schedule in which there are 48 cycles of 3min of optostimulation followed by 12min of no stimulation (total of 12 hours) after which there is 6 hours of no stimulation to allow flies to return to baseline.  DFM #2 is program to follow a “closed-loop” schedule in which optostimulation occurs in individual wells only when flies are interacting with the food (and generating a signal > 20, after baseline). The positive reinforcement lasts for 12 hours, after which there is 6 hours of no stimulation to allow flies to return to baseline.  All stimulation occurs at 40hz with 8ms pulses, and there is a 1 second decay that applies to the closed loop program (the LEDs will remain active for 1sec after each interaction).  There is no linkage, as this experiment was designed to use single-well chambers in all DFM.